USER   MANUAL

250 BEAM

Please keep read this manual carefully for further use!

Thank you for buying our professional Beam moving head.

This user manual includes important information for installation and operation, so please read this user manual carefully before installation and operation, and keep this user manual well for further use.

In order to install , operate, and maintain the lighting safety and correctly. We suggest that the installation and operation should be done by the verified technician and follow the instruction strictly.

Spare part includes:

| Name | QTY | Unit |
| --- | --- | --- |
| Fixture | 1 | PC |
| User manual | 1 | PC |
| Power cable | 1 | PC |
| DMX Cable | 1 | PC |
| Safety rope | 1 | PC |
| Mounting bracket | 2 | PCS |
| bracket | 2 | PCS |

Please check carefully that there is no damage caused by transportation. Should there be any, consult your local dealer.

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

CAUTION!

Keep this device away from rain and moisture!

Unplug mains lead before opening the housing!

Warning!

Silkscreen symbol on light

Damages caused by the disregard of this user manual are not subject to warranty. The dealer will not accept liability for any resulting defects or problems.

Installation

► This fixture is not suitable for direct installation on combustible material surface, please keep the fixture surface at least 0.5m from any combustible material.

► If you use the quick lock cam in hanging up the fixture, please make sure the quick lock fasteners turned in the quick lock holes correctly.

►The applicable temperature for the fixture is between -25°C to 45°C. Do not use the lighting under or above this temperature range.

►The truss for hanging the fixture should be able to hold 10 times weight of this fixture and no deformation after 1 hour hang.

►Please don’t stand under the fixture when install, uninstall, move or repair the fixture.

►Please ensure the light is connected correctly and invite professional technician to check and confirm the electrical data before installation.

►Please invite professional technician to check the fixture and installation each year.

Rigging：

Clamp

Bracket

Safety rope

Quick lock

Insert the quick-lock fasteners of the first holder into the respective holes on the bottom of the device.Tighten the quick-lock fasteners fully clockwise.

Fix the clamp on truss or other fixture bracket.

Pull the safety rope through the holes on the bottom of the base and fasten it on the truss or other fixing point.

Note:This step is quite important to ensure that fixture will not drop down for the clamp damage.

Product specification

1. Voltage：100-240V，50/60HZ；

2. Power Consumption：300W；

3. Lamp: 250W

4. Average lifetime:2000hours；   Color Temp: 8000K；

5.Pan：540°(16bit) electronic error correction；

6.Tilt：270° (16bit) electronic error correction；

7.Color wheel:13 colors+white

8.Gobo wheel:13 gobos+white

9.Prism: big-angle honeycomb prism

10.Frost and 6-color wheel effect；

11.  Electrical dimming：0%~100%；support strobe at various speeds, strobe color marco

12.  Beam angle：2.4º

13. Over-heat protection；Display：LED

14. IP rate:IP20；

15.Product size：327*268*490mm；

16.outer carton size: 405*370*53mm；

17.N.W：14.5KG；

Display and operation

Panel description:

| DMX | DMX signal indicator |
| --- | --- |
| AUDIO | Indicator light when AUDIO is triggered |
| Up | to select upward |
| Down | to select downward |
| Left | to select left / return |
| Right | to select right |
| ENTER | To Confirm |
| Time on the left side of the screen | The total time the system is in use (the display can be turned off in the standard settings) |
| Time on the right side of the screen | The total time of this power-on (can be turned off in the standard settings) |
| Vertical signal at the top of the screen | Current DMX signal connection signal strength Red: No signal. White: Current signal strength |
| Square icon in the upper right corner of the screen | Green: normal, no errors |
|  | Red: There is an error (click the icon or "Information Menu/Error Message/View" to view the error |

Display operation interface:

| D Standard | Default set | Enter? | Restore factory default parameters |
| --- | --- | --- | --- |
|  | Address | 001-512 | Set address code |
|  | DMX mode | Mode1 Mode2 Mode3 Mode4 | Select DMX control mode |
|  | Effect mode | No/Yes | effect disc automatically finds the shortest distance rotation operation |
|  | Started lamp | Off/on | Choose whether to turn on the bulb every time it is "on" or "off" |
|  | Switch lamp | Off/on | Turn on the light bulb manually |
|  | No signal | Clear Hold | Keep DMX value or clear DMX channel value when there is no DMX signal |
|  | UI Color | Green Blue Golden Gray | Choose screen menu color |
|  | Show time | No/Yes | Show running time timer on the main interface |
|  | Brightness | 000-255 | Adjust screen brightness |
|  | Screensaver | Off/On | Turn off the screen when there is no touch or key operation |
|  | XY encoder | No/Yes | Cancel or use XY automatic error correction function |
|  | X inversion | No/Yes | Select X axis forward or reverse operation |
|  | Y inversion | No/Yes | Select Y-axis forward or reverse operation |
|  | Focus inversion | No/Yes | Select the focus axis to run in the forward or reverse direction |
|  | Zoom inversion | No/Yes | Select the magnifying axis to run in the forward or reverse direction |
| Advanced | This setting is the main parameter setting of the lamp and requires permission |  |  |
| Info | Error List | No error/View | If there is no error or there is an error, click OK to view the error |
|  | System ver | Vxxxxxxx | System version information |
|  | Serial NO. | xxxxxxxx | Equipment factory number |
|  | SYS timer | 00000.0H | Total running time of the system (hours) |
|  | Run timer | 000:00 | Running time of this power-on (hours)） |
|  | Lamp timer | 00000.0H | The total time the bulb on (hours)） |
|  | Equip TEMP | 000 | The temperature of the main part of the equipment (need equipment support) |
|  | Head TEMP | 000 | The temperature of the lamp head of the device (requires device support) |
|  | FAN 1 Speed | 0000 RPM | fan 1 speed (requires equipment support) |
|  | FAN 2 Speed | 0000 RPM | fan 2 speed (requires equipment support) |
| perform | Run mode | Auto/Sound | Auto/Sound operation |
|  | Run speed | 255 | Auto operation speed |
|  | Run cross | 255 | Set sliding steps for Auto/Sound operation (device support is required) |
|  | Built-in 1 | Off/On | built-in test program 1 |
|  | Built-in 2 | Off/On | built-in test program 2 |
|  | User PRO 1 | Off/On | User self-programming program 1 |
|  | User PRO 2 | Off/On | User self-programming program 2 |
|  | User PRO 3 | Off/On | User self-programming program 3 |
|  | User PRO 4 | Off/On | User self-programming program 4 |
|  | Circle shape | Off/On | (Reserved) |
|  | Shpae shape | Off/On | (Reserved) |
|  | Shape range | 000 | (Reserved) |
|  | Sound DB | 000 | Adjust Sound control sensitivity |
| Program | (Explain in detail on the next page) |  |  |
| Reset | System reset to initial position |  |  |

Function Description

Manual control or programming function description:

1: This function can program up to 4 user programs, 4 programs can be run in series, that is to say, the 4 programs that have been programmed can be switched to on at the same time in the "Test Run" function, and program 1 is run first when running , Then to program 2, to program 3, then program 4, and then loop back to program 1, and so on and so on. Of course, you can also single-select operation.

2: Operations that can be achieved by programming function buttons:

Select program, read program, select step, delete step, manual or console input, copy current step, paste to selected step, clear current step, save to selected program space

Manual control:

Manual/Console Press the up, down, left, and right buttons and the OK button to select the mode button to the manual position.

Then operate the up, down, left, and right buttons and the OK button to make the corresponding channel red, and press the up and down buttons to manually control.

Manual control programming:

Select the PRO1-4 that needs to be saved, and after setting the channel value of one step through manual control, click the step (number button is the step) as shown in the figure, and press the up and down buttons to go to the next step. Then manually control and set the channel value, and the next step is and so on..

Delete step:

If there are too many programming steps, the following steps can be deleted. Select the step button and press the "Down" button to return to the previous step that is not needed. At this time, the number of the step is displayed as> 0XX. At this time, press and hold the OK button for 3 seconds, and the following steps can be deleted. The display becomes =0XX.

Copy step:

Press the copy button to copy all channel values ​​of the current step and temporarily store them in the memory pasteboard.

Paste step:

After copying, you can select any step, and paste the channel value of the pasteboard into the currently selected step channel.

Clear step:

Press the reset button to reset the channel value of the current step to zero.

Console control programming:

This device provides a more convenient console input programming function, and you can also use this function to enter the console program step by step to realize the function of copying the program programmed by the console.

As shown in the figure, select the manual/control console input mode as the console.

At this time, all channel values ​​are input by the DMX console, manual invalid, paste invalid, clear invalid, all channel values ​​display DMX values ​​in real time.

At this time, programming only needs console input, or the console runs the programmed steps in a single step, that is, enters the console program. This device only needs to click the next number and press the up arrow to realize programming.

Save the program:

After programming each step by manual input or console input, you can save this programming. Please make sure to select the save location PRO1--PRO4, please note that the saved step length is equal to the step value = 0XX, instead of seeing the step value> 0XX. Please delete the unnecessary steps later, refer to (Delete Steps", and then press Save.

Read the program and re-edit the program:

As shown in the figure, select the PRO1--PRO4 to be read out, then click the step button, and press the <Down> button to display "Read Out". Press the OK key at this time to read the selected program. The value displayed after reading is the maximum step value of the current program. If the program is empty, it will display as "Empty" or "No"

At this time, you can re-edit the program that has been read out. After editing, press Save to overwrite the current program, or select another PRO1--PRO4 space to save as.

Run the program:

If you need to run the programmed and saved program, please return to the "Test Run/Perform" function menu, click to select the corresponding user program, and switch to "On".

Fixed Gobo Wheel and color wheel

Six-color effect

DMX channel

| Channel | Effect | Value | Description |
| --- | --- | --- | --- |
| CH1 | Color | 0-3 | White |
|  |  | 4-7 | White+ color 1 |
|  |  | 8-11 | Color 1 |
|  |  | 12-15 | Color 1+ color 2 |
|  |  | 16-19 | color 2 |
|  |  | 20-23 | color 2+ color 3 |
|  |  | 24-27 | color 3 |
|  |  | 28-31 | color 3+ color 4 |
|  |  | 32-35 | color 4 |
|  |  | 36-39 | color 4+ color 5 |
|  |  | 40-43 | color 5 |
|  |  | 44-47 | color 5+ color 6 |
|  |  | 48-51 | color 6 |
|  |  | 52-55 | color 6+ color 7 |
|  |  | 56-59 | color 7 |
|  |  | 60-63 | color 7+ color 8 |
|  |  | 64-67 | Color 8 |
|  |  | 68-71 | Color 8+color 9 |
|  |  | 72-75 | Color 9 |
|  |  | 76-79 | Color 9+color 10 |
|  |  | 80-83 | Color 10 |
|  |  | 84-87 | Color 10+color 11 |
|  |  | 88-91 | Color 11 |
|  |  | 92-95 | Color 11+color 12 |
|  |  | 96-99 | Color 12 |
|  |  | 100-103 | Color 12+color 13 |
|  |  | 104-107 | Color 13 |
|  |  | 108-127 | Color 13+White |
|  |  | 128-191 | Rainbow effect forward rotation from slow to fast |
|  |  | 192-255 | Rainbow effect reverse rotation from slow to fast |
| CH2 | Strobe | 0-3 | Off |
|  |  | 4-103 | Strobe from slow to fast |
|  |  | 104-107 | On |
|  |  | 108-207 | Pulse strobe from slow to fast |
|  |  | 208-212 | On |
|  |  | 213-251 | Random strobe from slow to fast |
|  |  | 252-255 | On |
| CH3 | Dimmer | 0-255 | Dimmer from 0-100% |
| CH4 | Gobo | 0-4 | Gobo 1 |
|  |  | 5-9 | Gobo 2 |
|  |  | 10-14 | Gobo 3 |
|  |  | 15-19 | Gobo 4 |
|  |  | 20-24 | Gobo 5 |
|  |  | 25-29 | Gobo 6 |
|  |  | 30-34 | Gobo 7 |
|  |  | 35-39 | Gobo 8 |
|  |  | 40-44 | Gobo 9 |
|  |  | 45-49 | Gobo 10 |
|  |  | 50-54 | Gobo 10 |
|  |  | 55-59 | Gobo 12 |
|  |  | 60-64 | Gobo 13 |
|  |  | 65-69 | Gobo 14 |
|  |  | 70-74 | Gobo 1 shake from slow to fast |
|  |  | 75-79 | Gobo 2 shake from slow to fast |
|  |  | 80-84 | Gobo 3 shake from slow to fast |
|  |  | 85-89 | Gobo 4 shake from slow to fast |
|  |  | 90-94 | Gobo 5 shake from slow to fast |
|  |  | 95-99 | Gobo 6 shake from slow to fast |
|  |  | 100-104 | Gobo 7 shake from slow to fast |
|  |  | 105-109 | Gobo 8 shake from slow to fast |
|  |  | 110-114 | Gobo 9 shake from slow to fast |
|  |  | 115-119 | Gobo 10 shake from slow to fast |
|  |  | 120-124 | Gobo 11 shake from slow to fast |
|  |  | 125-129 | Gobo 12 shake from slow to fast |
|  |  | 130-134 | Gobo 13 shake from slow to fast |
|  |  | 135-139 | Gobo 14 shake from slow to fast |
|  |  | 140-194 | Reverse flow (from fast to slow) |
|  |  | 195-200 |  |
|  |  | 201-255 | Forward flow (from slow to fast) |
| CH5 | Prism | 0-127 | No function |
|  |  | 128-255 | Prism |
| CH6 | Prism Rotation | 0-127 | angle |
|  |  | 128-191 | Forward rotation from fast to slow |
|  |  | 192-255 | Reverse rotation from slow to fast |
| CH7 | Frost+6-color | 0-127 | angle |
|  |  | 128-191 | Forward rotation from fast to slow |
|  |  | 192-255 | Reverse rotation from slow to fast |
| CH8 | Focus | 0-255 | Focus |
| CH9 | Focus fine | 0-255 | Focus fine |
| CH10 | Pan | 0-255 | 0-540 degrees |
| CH11 | Pan fine | 0-255 | 0-2.1 degrees Pan fine |
| CH12 | Tilt | 0-255 | 0-270degrees |
| CH13 | Tilt fine | 0-255 | Tilt fine |
| CH14 | Pan/Tilt speed | 0-255 | Speed from fast to slow |
| CH15 |  | 0-255 | No function |
| CH16 | Lamp control | 0-99 | No function |
|  |  | 100-105 | Delay Off |
|  |  | 106-149 |  |
|  |  | 150-155 | Half-power On |
|  |  | 156-199 |  |
|  |  | 200-205 | Delay On |
|  |  | 206-249 |  |
|  |  | 250-255 | Reset after 3 sec |

DMX-512 connection

- Connect the provided XLR cable to the male 3-pin XLR output of your controller and the other side to the female 3-pin XLR input of the light.then connect the DMX signal to the first fixture to the 2nd fixture, after all fixtures are connected,

- The signal cable should be two - core with screened cable with XLR input and output connectors. Please refer to the diagram below

DMX Address setting

Each fixture must set a specific starting address. When receiving signal transmission, fixture will receive channel control signals which from the starting address.

According to different requirements or easy to use, the user can set many value fixtures to the same address, also can set up a separate address for every device.

If you set the same address,all the devices will receive from DMX channels signal. All connecting fixtures are jointly controlled, controller can't separate control a device.

If you set a different address, each device will be separate from its particular set the starting address and begin accepting DMX512 control signal, so that more convenient separate control one device. Using this method must be based on channel quantity of device to determine a starting address.

This light has 16 channels.you should set the starting address of the first unit to 1, the second unit to 17(1 + 16), the third unit to 33 (16*2+ 1), and so on.

Cleaning and maintenance

- Cleaning should be performed every 15-day period, by using a sponge which is dipped with alcohol, rather than wet cloth or other chemical liquid, to clean the mirror.

Cut off power before cleaning and maintenance!

CAUTION

Attention

Do not see directly light source when open the light, and make sure power off before do any installation or maintenance.

Wires connection must be done professional person!

CAUTION

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

Common issues instruction

| Issues | Solve instruction |
| --- | --- |
| No power | Check power’s voltage |
| When power in, output light but the device cannot accept control signal | Check address setting and the control DMX cable connection well or not |
| The effect have light shadow | Check electronic focus DMX channel value, does suitable distance |
| Power down | The lamp maybe arrived end of using life, should change new lamp, and also check light source cleaning or not |

Note:  the maintenance should be done by the verified technician.

Warranty Card This product is made of high-brightness transistor. We will provide 1 year warranty under the condition that user has operated the light normally and lifetime service. We won't provide warranty if the damage is caused by artificial or force majeure event. Cost of fittings should be charged by user if product need maintenance after 1 year .Please cut this warranty card and shipped it along with the product to our factory when applying for warranty.

Product： Purchasing date： Code of invoice：

Warranty date: From To

User’s name： Add：

Company： Tel：