BSW MOVING HEAD

USER MANUAL

Please read and keep this manual carefully for further use!

Thank you for buying our professional Beam moving head.

This user manual includes important information for installation and operation, so please read this user manual carefully before installation and operation, and keep this user manual well for further use.

In order to install , operate, and maintain the lighting safety and correctly. We suggest that the installation and operation should be done by the verified technician and follow the instruction strictly.

Spare part includes:

| Name | QTY | Unit |
| --- | --- | --- |
| Fixture | 1 | PC |
| User manual | 1 | PC |
| Power cable | 1 | PC |
| DMX signal cable | 1 | PC |
| Safety rope | 1 | PC |

Please check carefully that there is no damage caused by transportation. Should there be any, consult your local dealer.

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

CAUTION!

Keep this device away from rain and moisture!

Unplug mains lead before opening the housing!

Warning!

Silkscreen symbol on light

Damages caused by the disregard of this user manual are not subject to warranty. The dealer will not accept liability for any resulting defects or problems.

Installation

► This fixture is not suitable for direct installation on combustible material surface, please keep the fixture surface at least 0.5m from any combustible material.

► If you use the quick lock cam in hanging up the fixture, please make sure the quick lock fasteners turned in the quick lock holes correctly.

►The applicable temperature for the fixture is between -25°C to 45°C. Do not use the lighting under or above this temperature range.

►The truss for hanging the fixture should be able to hold 10 times weight of this fixture and no deformation after 1 hour hang.

►Please don’t stand under the fixture when install, uninstall, move or repair the fixture.

►Please ensure the light is connected correctly and invite professional technician to check and confirm the electrical data before installation.

►Please invite professional technician to check the fixture and installation each year.

Rigging：

Clamp

Bracket

Safety rope

Quick lock

Insert the quick-lock fasteners of the first holder into the respective holes on the bottom of the device.Tighten the quick-lock fasteners fully clockwise.

Fix the clamp on truss or other fixture bracket.

Pull the safety rope through the holes on the bottom of the base and fasten it on the truss or other fixing point.

Note:This step is quite important to ensure that fixture will not drop down for the clamp damage.

Product specification

1. Voltage：100-240V，50/60HZ；

2. Power Consumption：580W；

3. Lamp:Philips 380W

4.Fuse:8A/250V，￠5*20

5.DMX Channel: 24CH

6.Operation mode: master-slave/DMX/Auto

7. Focus/Zoom: Yes

8. Fixed gobo: 11 gobos+open+ Filter

Flame plate: 2+1+1 gobos

9. Strobe: 25Hz

10. Color: 12 colors+open

11. Prism 1: 8+16-facet

12. Prism 2:16-facet

13. Pan: 540°

14. Tilt: 270°

15.Frost / Six-color effect: Yes

16. RDM: Yes

17. Display: LCD touch screen

Display and operation

| Touch button | Function |
| --- | --- |
| Up | Option, Edit |
| Down | Option, Edit |
| Left | Backtrack, Edit |
| Right | Backtrack, Edit |
| OK | Enter to edit , confirm |

Setting:

| Main Menu | Menu 1 | Menu 2 |
| --- | --- | --- |
| DMX setting | DMX Channel | 24CH |
|  | DMX signal | Hold |
|  |  | Clear |
|  | RDM | On |
|  |  | Off |
|  | DMX monitor |  |
| Fixture test | Auto | Off |
|  |  | On |
|  | Manual | 1.Color 000-255 |
|  |  | 2.Shutter/strobe 000-255 |
|  |  | …… |
|  |  | 19.frost 000-255 |
|  | Reset |  |
|  | XY Reset |  |
|  | MT Reset |  |
| System Info | Temperature | Display panel temperature |
|  |  | Motor board temperature |
|  | Fan | Fan speed |
|  | system version |  |
|  | System time | Total usage time |
|  |  | time of this use |
|  |  | Total lamp-on time |
|  |  | Time of this lamp-on |
|  | Sensor monitoring | X hall |
|  |  | Y hall |
|  |  | color hall |
|  |  | Pattern Hall |
|  |  | effect pattern hall |
|  |  | Glass gobo hall |
|  |  | Focus Hall |
|  |  | Zoom Hall |
|  |  | Prism 1 Hall |
|  |  | Prism 2 Hall |
|  |  | X encoding status |
|  |  | Y encoding status |
|  |  | X encoding step value |
|  |  | Y encoding step value |
|  | system error |  |
| Fixture setting | Language | English |
|  |  | Chinese |
|  | screen flip off | off |
|  |  | on |
|  | screen saver | on |
|  |  | off |
|  | X reverse | off |
|  |  | on |
|  | Y reverse | off |
|  |  | on |
|  | XY swap | off |
|  |  | on |
|  | XY encoder | on |
|  |  | off |
|  | Linear change of color wheel | on |
|  |  | off |
|  | reset |  |
| Factory setting (The password is "up up down down". ) | Motor Calibration | X |
|  |  | Y |
|  |  | Color |
|  |  | Pattern |
|  |  | effect pattern |
|  |  | Glass gobo |
|  |  | focus |
|  |  | zoom |
|  |  | Dimming zero |
|  |  | Dimming stroke |
|  |  | Prism 1 Zero |
|  |  | Prism 1 stroke |
|  |  | Prism 2 Zero |
|  |  | Prism 2 stroke |
|  |  | Forst stroke |
|  | Stroke calibration | X stroke |
|  |  | Y stroke |
|  |  | Focus stroke |
|  |  | Zoom stroke |
|  | XY speed adjustment | X speed |
|  |  | Y speed |
|  | Fan adjustment | White-on fan |
|  |  | Color-on fan |
|  |  | White-off fan |
|  |  | Color-ff fan |

24 DMX channel

| 24CH | Function | Value | Details |
| --- | --- | --- | --- |
| CH1 | Color wheel | 000-004 005 -009 010 - 014 015 - 019 020 - 024 025 - 029 030 - 034 035 - 039 040 - 044 045 - 049 050 - 054 055 - 059 060 - 064 065 - 069 070 - 074 075 - 079 080 - 084 085 - 089 090 - 094 095 - 099 100 -104 105 -109 110 -114 115 -119 120 -124 125-129 130 -192 193 - 255 | White White+Color 1 Color 1 Color 1+color 2 Color 2 Color 2+color 3 Color 3 Color 3+color 4 Color 4 Color 4+color 5 Color 5 Color 5+color 6 Color 6 Color 6+color 7 Color 7 Color 7+color 8 Color 8 Color 8+color 9 Color 9 Color 9+color 10 Color 10 Color 10+color 11 Color 11 Color 11+color 12 Color 12 Color 12+White Forward flow (from fast to slow) Reverse flow (from slow to fast) |
| CH2 | Shutter /strobe | 000-003 004-250 251-255 | Shutter closed Strobe from slow to fast Open the shutter → (controlled by the dimming channel) |
| CH3 | dimmer | 000-255 | dimmer from dark to bright |

| CH4 | gobo wheel | 000 - 004 005 - 009 010 - 014 015 - 019 020 - 024 025 - 029 030 - 034 035 - 039 040 - 044 045 - 049 050 - 054 055 - 059 060 - 064 065 - 069 070 - 074 075 - 079 080 - 084 085 - 089 090 - 094 095 - 099 100 - 104 105 - 109 110 - 114 115 - 119 120 - 187 188 - 255 | gobo 1 gobo 2 gobo 3 gobo 4 gobo 5 gobo 6 gobo 7 gobo 8 gobo 9 gobo 10 gobo 11 gobo 12 gobo 1 shake slow to fast gobo 2 shake slow to fast gobo 3 shake slow to fast gobo 4 shake slow to fast gobo 5 shake slow to fast gobo 6 shake slow to fast gobo 7 shake slow to fast gobo 8 shake slow to fast gobo 9 shake slow to fast gobo 10 shake slow to fast gobo 11 shake slow to fast gobo 12 shake slow to fast Reverse flow (from fast to slow) Forward flow (from slow to fast) |
| --- | --- | --- | --- |
| CH5 | Effect gobo | 000 - 009 010 - 019 020 -029 030 - 114 115 - 134 135 - 154 155 - 255 | gobo 1 (blank) gobo 2 gobo 3 gobo 4 (water pattern) gobo 2 shake slow to fast gobo 3 shake slow to fast gobo 4 (water pattern) slow to fast |
| CH6 | Glass gobo | 000 - 009 010 - 019 020 -029 030 -039 040 - 049 050 -059 060 - 069 070 - 079 080 - 089 090 - 099 100 - 109 110 - 119 120 - 129 130 - 139 140 - 149 150 - 200 201 - 255 | gobo 1 gobo 2 gobo 3 gobo 4 gobo 5 gobo 6 gobo 7 gobo 8 gobo 2 shake slow to fast gobo 3 shake slow to fast gobo 4 shake slow to fast gobo 5 shake slow to fast gobo 6 shake slow to fast gobo 7 shake slow to fast gobo 8 shake slow to fast Forward flow (from fast to slow) Reverse flow (from slow to fast) |
| CH7 | Glass gobo rotation | 000-127 128-191 192-255 | Angle adjustment Forward rotation fast to slow Reverse rotation slow to fast |
| CH8 | Prism 1 | 000-127 128-255 | no effect prism 1 |
| CH9 | Prism1 rotation | 000-127 128-191 192-255 | prism angle adjustment Reverse rotation fast to slow Forward rotation slow to fast |
| CH10 | Prism 2 | 000-127 128-255 | no effect prism 2 |
| CH11 | Prism2 rotation | 000-127 128-191 192-255 | prism angle adjustment Reverse rotation fast to slow Forward rotation slow to fast |
| CH12 | Zoom | 000-255 | Pattern clarity from far to near |
| CH13 | focus | 000-255 | Pattern clarity from far to near |
| CH14 | pan | 000-255 | 0-540 degree |
| CH15 | pan fine | 000-255 | 0-1.2 degree |
| CH16 | tilt | 000-255 | 0-270 degree |
| CH17 | tilt fine | 000-255 | 0-1.2 degree |
| CH18 | XY speed | 000-255 | speed fast to slow |
| CH19 | frost | 000-127 128-255 | no function frost |
| CH20 | Lamp control & Reset | 000-099 100-105 200-205 250-255 | no function Lamp off Lamp on all motors reset |
| CH21 | Hold | 000-255 |  |
| CH22 | Color speed | 000-255 | Speed from slow to fast |
| CH23 | Dimmer speed | 000-255 | Speed from slow to fast |
| CH24 | gobo speed | 000-255 | Speed from slow to fast |

Gobo wheel                                 Flame plate

Color wheel                               Rotation wheel

DMX-512 connection

- Connect the provided XLR cable to the male 3-pin XLR output of your controller and the other side to the female 3-pin XLR input of the light.then connect the DMX signal to the first fixture to the 2nd fixture, after all fixtures are connected,

- The signal cable should be two - core with screened cable with XLR input and output connectors. Please refer to the diagram below

DMX terminator connection

If there are many lines or lights, it is recommended to use a DMX terminator to prevent DMX signal corruption, the DMX terminator is an simple XLR plug with a 120 Ω resistor connected between pins 2 and 3,which is then plugged into the output XLR socket of the last fixture in the chain.

3-Pins connection

Cleaning and maintenance

- Cleaning should be performed every 15-day period, by using a sponge which is dipped with alcohol, rather than wet cloth or other chemical liquid, to clean the mirror.

Cut off power before cleaning and maintenance!

CAUTION

Attention

Do not see directly light source when open the light, and make sure power off before do any installation or maintenance.

Wires connection must be done professional person!

CAUTION

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

Common issues instruction

| Issues | Solve instruction |
| --- | --- |
| No power | Check power’s voltage |
| When power in, output light but the device cannot accept control signal | Check address setting and the control DMX cable connection well or not |
| The effect have light shadow | Check electronic focus DMX channel value, if it’s suitable distance |
| Light beam dim | The lamp maybe arrived end of using life, should change new lamp, and also check light source cleaning or not |

Note:  the maintenance should be done by the verified technician.

Warranty Card This product is made of high-brightness transistor. We will provide 1 year warranty under the condition that user has operated the light normally and lifetime service. We won't provide warranty if the damage is caused by artificial or force majeure event. Cost of fittings should be charged by user if product need maintenance after 1 year .Please cut this warranty card and shipped it along with the product to our factory when applying for warranty.

Product： Purchasing date： Code of invoice：

Warranty date: From To

User’s name： Add：

Company： Tel：